#### ENGINE FIRE - STARTING

Starter 		crank engine

Mixture 		idle cut-off

Throttle 				open

Electric fuel pump	OFF

Fuel selector 		OFF

#### Abandon if fire continues

##### ENGINE OUT IN FLIGHT

Fuel selector 	switch to

tank with fuel

Electric fuel pump	ON

Mixture 				RICH

Carb heat 			ON

Engine gauges	check for

cause of power loss

Primer	 				locked

Fuel pressure 	if no fuel

pressure check

fuel selector

Trim for 			73 KIAS

#### Power Restored:

Carb heat 			OFF

Electric fuel pump	OFF

### POWER OFF LANDING

Trim for 			73 KIAS

Glide range 	    10 x altitude

Locate suitable landing site

Manuver to landing site

At 1000 AGL 	63 KIAS

Full flaps

#### When committed to landing:

Ignition 				OFF

Master Switch 		OFF

Fuel selector 		OFF

Mixture 			idle cut-off

Seat belts/harness 	tight

Touchdown 			slowest

### FIRE IN FLIGHT

Source of fire 			check

Smoke in cabin (Electrical fire):

Master Switch 			OFF

Vents 					Open

Cabin heat 				OFF

Land as soon as possible

Engine fire:

Fuel selector 			OFF

Throttle 					Closed

Mixture 					idle cut-off

Electric fuel pump 		OFF

Heater 					OFF

Defroster 				OFF

Proceed to POWER OFF LANDING

### LOSS OF OIL PRESSURE

Land ASAP and investigate

Proceed to POWER OFF LANDING

LOSS OF FUEL PRESSURE

Electric fuel pump 		ON

Fuel selector 			Fullest

### HIGH OIL TEMPERATURE

Land ASAP and investigate

Proceed to POWER OFF LANDING

ALTERNATOR FAILURE

Verify failure

Reduce electrical load

Alt circut breakers 	check

Alt switch 			cycle

OFF one second then ON

# SPIN RECOVERY

Throttle 				idle

Ailerons 			neutral

Rudder 				Full opposite

to rotation

Yoke 				Full forward

Rudder 				neutral when

rotation stops

Yoke 				regain level

flight

# OPEN DOOR

If both upper & lower latches are open, door will trail slightly open and airspeed will be reduced

To close door in flight:

Airspeed 				89 KIAS

Cabin vents 			closed

Storm window 			open

Close upper latch

Pull arm rest - close lower latch

ENGINE ROUGHNESS

Carb heat 				ON

If roughness after 1 minute:

Carb heat 				OFF

Mixture 			adjust for smooth

Electric fuel pump 		ON

Fuel selector 		   switch tanks

Engine gauges 	   check

Magneto 	L then R then BOTH

Proceed on best Magneto

Proceed to nearest airport

Proceed to POWER OFF LANDING

CARBURETOR ICING

Carb heat 				ON

Mixture 				adjust

# LIGHT SIGNALS - Ground

Steady GREEN – Cleared for 					     Takeoff

Flashing GREEN – Cleared for 					       Taxi

Steady RED – STOP!

Flashing RED – Taxi clear of 					  runway

Flashing WHITE – Return to    	     starting point on airport

Alternating RED/GREEN – 			Exercise Extreme Caution

# LIGHT SIGNALS - Flight

Steady GREEN – Cleared to 					     Land

Flashing GREEN – Return for 					       Landing

Steady RED – Give way & 					     circle

Flashing RED – DO NOT land

Flashing WHITE (Not used)

Alternating RED/GREEN – 			Exercise Extreme Caution