#### V-SPEEDS  (KIAS)

VR (No Flaps)		55

VR (10º Flaps)		50

VR (25º Flaps)		47

VX (No Flaps) 		63

VX (25º Flaps) 		52

VY 						79

VGlide 					73

VS0  (landing config)	44

VS1 	(clean)		50

VFE 					103

VA 						111-88

VNO 					126

VNE 					160

##### GROUND CHECK

Cabin doors 			secure

Seats / belts			secure

Master switch 		ON

Avionics 				ON

Primer 				locked

Fuel selector 		fullest

Flight controls 		check

Flight instruments 	check

### RUN UP

Throttle 			2000 RPM

Magnetos 			check

Max drop 			175 RPM

Max diff 			50 RPM

Vacuum 				5” ± 1”Hg

Oil pressure 			check

Oil temperature 		check

Ammeter 				check

Air conditioner 		OFF

Annunciator panel 	test

Carburetor heat 		check

Fuel pump 			OFF

Throttle 			1000 RPM

### TAKEOFF, BEFORE

Fuel selector 			fullest

Flaps 					0-25º

Elevator trim 			takeoff

Mixture 					RICH

Carb heat 				OFF

Fuel pump 				ON

Transponder 			ALT

Radio 					check

### TAKEOFF, NORMAL

Flaps 					10º

VR 						45-55 KIAS

VY 						79 KIAS

Flaps 					retract

### TAKEOFF, SHORT-FIELD

Flaps 					25º

VR (Clear obstacle)	52 KIAS

VY 						79 KIAS

Flaps 					retract

### TAKEOFF, SOFT-FIELD

Flaps 					25º

Lift nose 				ASAP

VR 						47 KIAS

VX (Clear obstacle)	52 KIAS

VY 						79 KIAS

Flaps 					retract

# CLIMB

(VY) Best rate 			79 KIAS

(VX) Best angle 		63 KIAS

En route 				87 KIAS

Fuel pump 				OFF

CLOSE FLIGHTPLAN

# DESCENT, NORMAL

Throttle 					1500 RPM

Airspeed 				111 KIAS

Mixture 					RICH

Carb heat 				as needed

# APPROACH, DOWNWIND

Fuel selector 			fullest

Mixture 					RICH

Carb heat 				cycle

Fuel pump 				ON

Air conditioner 			OFF

Throttle 					1500 RPM

Airspeed in white arc <103 KIAS

Altitude					1000 AGL

Flaps					10º

Trim to 					80 KIAS

# APPROACH, CROSSWIND

# Altitude					800 AGL

# Flaps 					25º

# Trim to 					70 KIAS

# APPROACH, FINAL

Altitude					500 AGL

Flaps 					40º

Trim to 					63 KIAS

# AFTER LANDING

Flaps 					retract

Carb heat 				OFF

Fuel pump 				OFF

Transponder 			STBY

# ENGINE STOP

Air conditioner 			OFF

Avionics 				OFF

Throttle 					closed

Mixture 					cutoff

Ignition switch 			OFF

Master switch 			OFF

# SECURING

Parking brake 			set

Control lock 			set

Wheel chocks 			set

Tie downs 				secure

###### CLOSE FLIGHTPLAN