# CABIN

ARROW 				check

Control lock 			release

Hobbs 				record

Tachometer 			record

Avionics 				OFF

Ignition switch 		OFF

Master switch 		ON

Fuel gauges 			check

Lights 					set

Master switch 		OFF

Fuel selector 		fullest

Flaps 					10o

Parking brake 		set

# RIGHT WING

Flap / Aileron			check

Wing tip 				check

Tie down 				untie

Fuel quantity 		check

Fuel cap 				secure

Fuel vent 				open

Fuel sump 			drain

# RIGHT MAIN GEAR

Main strut 			check

Tire / Brakes 			check

Chock 				remove

# NOSE

Windshield 			clean

Oil quantity 			6-8 qts

Inspection covers 	check

Right Cowling 		secure

Prop & spinner 		check

Air inlets 				clear

Alternator belt 		tension

Landing light 			check

Inspection covers 	check

Left Cowling 			secure

Fuel sump 				drain

# NOSE GEAR

Strut 					check

Tire /Brakes 			check

Chock 					remove

# LEFT MAIN GEAR

Main strut 				check

Tire / Brakes			check

Chock 					remove

# LEFT WING

Fuel quantity 			check

Fuel cap 				secure

Fuel vent 				open

Fuel sump 				drain

Tie down 				untie

Pitot tube cover 		remove

Wing tip 				check

Aileron / Flap			check

# EMPENNAGE

Antennas 				check

Fuselage 				check

Tail lights 				check

Rudder 					check

Elevator 				check

# BAGGAGE

Tow bar 				secure

Control Cables 		secure

Baggage door 			secure

# BEFORE STARTING

Parking brake 			set

Carburetor heat 		OFF

Fuel selector 			fullest

Avionics 				OFF

Circuit breakers 		IN

# COLD / HOT START

Prime (cold) 			3-4

Throttle 					¼” open

Master switch 			ON

Fuel pump 				ON

Mixture 					RICH

Starter 					engage

Throttle 					1000 RPM

Oil pressure 			check

# FLOODED START

Throttle 					FULL

Master switch 			ON

Fuel pump 				OFF

Mixture 					cutoff

Starter 					engage

Mixture 					advance

Throttle 					retard

Oil Pressure 			check

EXTERNAL POWER START

Master switch 			OFF

Avionics 				OFF

Terminals 				connect

External power plug insert

Proceed with normal start

Throttle 					lowest

Extern power plug disconnect

Master switch 			ON

Oil pressure 			check